|  | Question | Answer |
| --- | --- | --- |
| 1. | Outline the design features and manufacturing techniques that have made Alessi successful | - Unique products produced with a distinctive design flavour. - Heavy focus on kitchen products and products for the household - Products make extensive use of stainless tell mixed with brightly coloured polymers. - Artistic design - Hand crafted objects made with the help of machines - Fun design - Aesthetic and functional with varying degrees of success |
| 2. | Explain what is meant by the term smart material | Composites are produced by combining 2 or more materials together to make use of their individual properties in the creation of a new enhanced material. |
| 3. | What is the definition of a composite | Smart materials are those who physical properties react to a change in their environment. |
| 4. | List a composite used for tennis racquets | Carbon fibre – Makes them light but strong |
| 5. | Which smart materials can be altered using heat? | Thermochromic polymers, shape memory alloys, polymorph |
| 6. | Explain why carbon fibre reinforced plastic (CRP) is used in sports equipment, motorsport vehicles and safety equipment such as helmets | It is lightweight and has an exceptionally high strength to weight ratio.​ It is stiff, rigid and tough and has good tensile strength. |
| 7. | Where might you use the modern material titanium? | Replacement body parts, surgical equipment |
| 8. | Which is the thermoset polymer used for electrical plugs? ​ | Urea formaldehyde |
| 9. | ​ Which thermoset polymer is used on kitchen worktops to protect the chipboard underneath? ​ | Melamine formaldehyde |
| 10. | Which thermosetting polymer is used in the composite GRP?​ | Polyester resin |
| 11. | ​Which thermoforming polymer is used for washing up bowls? | High density Polyethylene (HDPE) |
| 12. | What are the 3 categories of metal? | Alloys, ferrous, non-ferrous |
| 13. | What is the definition of malleability? | The ability of a material to deform without cracking |
| 14. | What is the definition of an alloy? | An alloy is a metal combined with other substances resulting in superior properties such as; strength, hardness, durability, ductility, tensile strength and toughness. |
| 15. | What is a disadvantage of cotton? | Creases easily |
| 16. | Name a clothing item that can be made from silk? | Shirt, tie, dress, poor insulator, shrinks |
| 17. | State 2 properties of cotton that make it suitable for a shirt | Absorbent, strong, cool to wear, hard wearing, smooth, easy to care for |
| 18. | Name a synthetic which can be blended with cotton to make an improved fabric | polyester |
| 19. | Give an example of a non-ferrous metal | Aluminium, Tin, copper, zinc |
| 20. | What makes steel? | Carbon and Iron |

OBHS Core Questions:

Subject: Design & Technology

Year and Term: Year 10 Spring term

Topic: Materials

Learn these questions to build a strong foundation of knowledge for this half-term. Ask family or friends to test you regularly, or practise on your own using the ‘Look, Say, Cover, Write’ method.