|  | Question | Answer |
| --- | --- | --- |
| 1. | What is felling? | A tree is ‘felled’ when it is cut down. |
| 2. | Once the trees are felled they are transported to a sawmill to be converted (changed) into timber planks and boards. What are the two main methods of conversion? | Through and through, quarter sawn |
| 3. | What are the 2 main methods of seasoning or drying put timber? | Kiln, air |
| 4. | List 2 advantages of Kiln drying? | - Quick - Easy to control - Kills insects and bacteria |
| 5. | List 2 disadvantages of kiln drying? | - Expensive equipment, uses a lot of energy |
| 6. | List 2 advantages of air drying? | - Cheap - No energy required |
| 7. | List 2 disadvantages of air drying? | - Difficult to control - Takes along time |
| 8. | What is shrinkage in timber? | when the timber is allowed to dry out in an uncontrolled way. It can twist, warp, cup and/or bow​ |
| 9. | Give 3 problems caused by knots in timber? | - they can weaken the timber ​ - they can lead to decay ​ - they can release resin which makes applying a finish to the wood very difficult. |
| 10. | What are the 2 methods of joining timber? | Temporary, permanent joints |
| 11. | Give 3 common permanent wood joints used in box or carcase construction? | Butt, finger, dovetail, lapped, housing joints |
| 12. | Give 2 common wood joints used for ‘stool’ construction | Mortise and tenon, Bridle joint, dowel joint |
| 13. | What are the 3 holes needed to fit a screw in timer correctly? | Pilot, clearance, countersink |
| 14. | Why do we use KD fittings with flat packed furniture? | - ‘KNOCK DOWN’ fittings are often used with flat packed furniture.​ - The enable furniture to be sold unassembled, taken home in a cardboard box and assembled using simple tools. ​ - The concept of flat packed furniture has greatly reduced the cost of buying furniture. ​ - Traditionally, a family would invest in a piece of furniture which would often be handed down through the generations. Now people can afford to replace furniture as fashion and style changes. |
| 15. | Explain why a bicycle is made in batch production? | - Changes in fashion or technology developments means that the mountain bike style may need to evolve - Mountain bike riders want value for money - Many parts of the bike are standard with other bikes - Machines in batch production can change set up easily |
| 16. | 100 tables are made. Which is the most suitable method of production for this? | Batch |
| 17. | Quality control checks are made to assess the quality of wooden products as they are being made. List some quality checks. | - Wood should be initially checked for any defects​ - The moisture content should be checked to ensure its suitability for its purpose​ - Regular measurements should be taken to ensure that the product is being made to the correct size.​ - Visual checks should be made to see if the surface is smooth and free from any unwanted marks. ​ - The use of templates, moulds, formers, fences and depth-stops and the use of CNC equipment will help increase the accuracy and consistency of a product. |
| 18. | What is a wood preservative? | A wood preservative soaks into the timber and protects it from both moisture and insect attack. Preservatives can also include a stain |
| 19. | Name the process of preserving wood? | Tanalising |
| 20. | List 3 timber finishes | Wax, varnish, paint, wood stain, oil, veneers |

OBHS Core Questions:

Subject: Design & Technology

Year and Term: Year 11 Spring term

Topic: Working with timber

Learn these questions to build a strong foundation of knowledge for this half-term. Ask family or friends to test you regularly, or practise on your own using the ‘Look, Say, Cover, Write’ method.